# xmitter Rev D Fix Implementation Plan

Oct 23, 2026 · @Al

## Scope and how to use this

This is the complete to-do list to take the Rev D schematics from the 2026-10-23 netlists to layout-ready. It consolidates every open item from review rounds 1 and 2 (M, S and N items) plus what ss_pa.md and front_panel_interface.md revealed, in the order you would work the schematics: one board at a time, then the harness, then the docs. Tick the boxes as you go; values are given wherever a part changes.

What the two supporting docs added:

- front_panel_interface.md pin map is right and the netlist is wrong: pin 5 is meant to be GND (paired with +5 V on pin 4). That is the M11 fix. The ENC_INT machinery (R102/J50 on analog, D4 interrupt) was designed for the dropped seesaw encoders; the doc itself concludes polling at 50 Hz and no INT cable. Delete the ENC_INT path everywhere.

- Both docs specify single-point shield grounding (ss_pa: PA end only; frontpanel: analog end only), but every RJ45 in every netlist ties SH to GND at both ends. Pick one policy (see Decisions).

- ss_pa.md describes a differential J3/J4 drive straight into the IRF510 gates with no BS170 driver and no T201; the netlist has single-ended J3 → T201 → BS170 pair → T202. The netlist is the better design; the doc is a Rev C snapshot. Its "<1 µs response" claim is the 10 ms M8 problem.

- ss_pa.md plans 0.1 µF bypass across the 0.11 Ω source-sense resistors at layout. At 14 MHz 0.1 µF is 0.11 Ω, so it only halves the RF on the sense node; use 1 µF and still add the comparator RC (P2).

- front_panel_interface.md says the Metro is USB-powered and that the RJE1D-188 footprint was flagged wrong on 2026-07-15 pending a Rev A fit check. Both are open decisions below.

## Analog board (KiCAD/analog/)

Work vfo_complete first (A1–A4), then power, dac, arduino, interface.

A1 — AC-couple the DGFET into U1 (N2). On vfo_complete:

- ☐ Insert a 10 nF C0G between the Q2 drain / R31 node and R1 pin 1. Drain DC never reaches U1.

- ☐ Add 10 k from U1 pin 3 to GND (or to the +6 V node in A3 if you take the single-supply option).

- ☐ Keep R1 50 Ω, L1 270 nH, C1 120 pF as they are; the 28 MHz / 47 Ω filter is correct.

A2 — Make the DGFET actually cut off at key-up (N3 / M3). Revised 2026-10-23 after simulation. The G1-divider scheme first written here does not work: with G1 at +2.45 V and R9 to ground, muting G2 to 0 V leaves ≈ 5 mA flowing (on/off ratio 2.6:1). Any gate referenced to ground cannot cut the device off while the source sits above ground; the mute clamp has to go below ground. Pull G2 to −3 V at key-up (simulated: I_D ≈ 16 pA, drain at +12.0 V). This keeps the −12 V rail (see A3).

- ☐ Keep the G1 divider from the first draft (39 k / 10 k from +12 V, 100 nF, R21 to G1_BIAS) and R9 220 Ω — it sets the keyed point: I_D ≈ 13 mA, source ≈ 2.9 V, drain ≈ 5.9 V.

- ☐ Make a stiff −3 V node VNEG3: 1N4728A zener, cathode to GND, anode to VNEG3; 820 Ω from VNEG3 to −12 V (≈ 11 mA bias). 100 nF VNEG3 to GND.

- ☐ Rewire the mute as one NPN. Q11 emitter → VNEG3, collector → Gate 2 (as now). Q11 base from a divider between KeyUp and −12 V: 3.3 k KeyUp → base, 6.8 k base → −12 V. KeyUp high (5 V) → base ≈ −0.6 V, Q11 on, G2 clamped to ≈ −2.9 V; KeyUp low → base ≈ −3.9 V, Q11 off.

- ☐ Delete Q12, R48, R49, R50. Keep R51 (KeyUp pull-up to +5 V: PCF8575 power-on state = muted) and R13 1 k.

- ☐ Firmware zeroes the MCP4922 G2 channel at key-up as well; otherwise the DAC sources ≈ 8 mA into the clamp through R13 (within its 15 mA rating, but pointless).

- ☐ Re-run the QUCS-S check with the clamp: key-up I_D < 1 µA, V_G2S ≤ −2.5 V.

Alternative if you ever drop −12 V: leave Q11/Q12 as drawn and add a BS170 in series between the bottom of R9 and GND, gate driven from Q11's collector through a 10 k pull-up to +5 V (so Q11 becomes an inverter: KeyUp high → FET off → no DC path). Not simulated; the −3 V clamp above is.

A3 — Keep the −12 V rail (reverses the round-1 S1 suggestion). A2 needs a negative clamp, so the LT1054 stays. Fix its regulation instead:

- ☐ R16 20 k / R17 240 k target ≈ −16 V, which the inverter cannot reach from 13.8 V. Either leave FB/SHDN open (unregulated, ≈ −12.5 V light load) or set R17 = 160 k for ≈ −10 V regulated. −10 V is enough for U1 and the A2 clamp.

- ☐ Budget: U1 negative-rail current (up to ~80 mA keyed) + 11 mA for VNEG3 + clamp current is close to the LT1054's 100 mA. Keep U1 at unity gain unless measured drive demands more.

- ☐ U1 pin 3 bias stays at GND via the 10 k from A1; output DC ≈ 0 V, ±10 V swing available.

A4 — SW1 (N4).

- ☐ Replace SW1 with a 2×2 0.1" header + two shunts (SDA bus ↔ U5 SDA, SCL bus ↔ U5 SCL). If you keep the slide switch, bench-verify with a meter that pins 1–2 and 5–6 close in the same slider position before layout.

A5 — Missing part data (N5). On power and vfo_complete:

- ☐ D1: value 1N5822, footprint Diode_THT:D_DO-201AD_P15.24mm_Horizontal. Give it a 2 oz pour on both leads; ~1.5 W at 3 A.

- ☐ L1: assign a footprint (axial, or xmitter:T50-6_Flat_HandWound with 8 turns).

A6 — 74HCT125 inputs (N6). On dac:

- ☐ 10 k pull-down to GND on U9 pins 2 (MOSI), 5 (SCK), 12 (DAC_LDAC_N).

- ☐ 10 k pull-up to +3.3 V on U9 pin 9 (CS_DAC) so the DAC is deselected with the Metro unplugged.

A7 — Encoder quadrature level shift (M14). On interface:

- ☐ R28, R29: 10 k → 3.3 k. High level becomes 5 × 3.3/5.5 = 3.0 V; the protoshield zeners and 100 nF then go (PS3).

A8 — Frontpanel umbilical ground (M11 / M13). On interface and arduino:

- ☐ J4 pin 5: ENC_INT_CLEAN → GND.

- ☐ Delete R6, J9 and the nets ENC_INT / ENC_INT_CLEAN.

A9 — Housekeeping on arduino:

- ☐ Add a 1N4148 flyback across the T/R relay output J6 (cathode to +12 V, anode to Q4 drain).

- ☐ Delete the orphan net PCF_P5_SPARE on J14 pin 4, or wire it to a spare PCF8575 port.

- ☐ Rename C8 U9 bypass and place it at U9 pin 14 during layout.

- ☐ J16 pin order → GND, SCK, MOSI, CS_DAC, DAC_LDAC_N (matches PS5).

A10 — Power (S4, S5, S6). On power:

- ☐ R7 (K5 NC pre-charge): keep 750 Ω but rate it 1 W, and note in revD_interconnect.md that J34 must be paralleled with J33 at ss_pa J2; firmware waits ≥ 10 s after power-up before closing K5. Or change to 100 Ω 2 W and wait 1 s.

- ☐ U6 LM2940CT-5.0: add a heatsink footprint. Budget ≈ 0.4 A → 3.5 W.

- ☐ Layout rule: PA return current (J33 pin 1) goes to J5 pin 1 on its own 2.5 mm copper, not through the analog ground pour. K5 contacts and the J5→F2→D1→K5→J33 path at ≥ 2.5 mm, 1 oz.

- ☐ Decide the Metro power path (see Decisions) and label J21 accordingly.

## ss_pa board (KiCAD/ss_pa/)

All changes are on fault_shutdown except P6 and P7.

P1 — U13 LM2936Z-5.0 (N1 / M4).

- ☐ Make an LM2936Z symbol in xmitter.kicad_sym with the pin order from the TI datasheet (TO-92 drawing, front view) and swap it in for the MCP1700 symbol; fix the Datasheet and Description fields. Do not reuse the MCP1700 pin map.

- ☐ Output capacitor: add 22 µF electrolytic on +5 V (LM2936 needs ≥ 10 µF; C21 1 µF is not enough). Keep C14/C15/C21.

- ☐ Input: C16 1 µF at pin VIN is fine; add 100 nF right at the pin during layout.

P2 — Filter the source-sense nodes before the comparator (M9).

- ☐ New R20 1 k from Q3 source node to a new node SENSE_A_F; new C22 10 nF SENSE_A_F to GND; U1 pin 2 (−in) → SENSE_A_F.

- ☐ Same for channel B: R21 1 k, C23 10 nF, SENSE_B_F → U1 pin 6.

- ☐ Move J31 pins 1 and 4 from the raw source nodes to SENSE_A_F / SENSE_B_F. The umbilical carries DC + envelope, not 14 MHz.

- ☐ On pa_stage: 1 µF X7R across R1∥R2 and across R3∥R4, short leads, component side (replaces the 0.1 µF planned in ss_pa.md).

P3 — Give the latch margin (M10).

- ☐ 220 k from +5 V to SENSE_A_F and 220 k from +5 V to SENSE_B_F. With the 1 k in P2 this floors the −in at ≈ 23 mV, so a tripped latch (V_REF = 0 V) can never release on offset.

P4 — Fast bias kill (M8).

- ☐ R12, R19: 100 k → 2.2 k. Gate-node decay after the 4066 opens becomes ≈ 220 µs instead of 10 ms. MCP4728 drives 2.2 k with 5 V easily.

- ☐ Firmware still zeroes PA_Bias on FAULT_N; hardware now covers the interval before the I²C write lands.

P5 — Thresholds and levels (keep as is, record them).

- ☐ R9 stays 4.7 k, not 10 k: with analog's R3/R4 divider loading it, 4.7 k gives 4.3 V at FAULT_N (HC4066 V_IH 3.5 V); 10 k would give 3.75 V.

- ☐ V_REF = 5 × 6.8/106.8 = 0.318 V → trip at ≈ 2.9 A average per device after P2. This is a gross-fault trip, not a bias servo; fine.

P6 — Mechanical (S12).

- ☐ Q3/Q4 tabs are drains at opposite RF phase: insulated pads + shoulder washers on the shared sink, called out on the assembly drawing.

P7 — Reset path.

- ☐ Keep J5. Fit a momentary pushbutton across it. Optionally wire a Metro GPIO through 1 k to J5 pin 2 so firmware can auto-clear after BIST (the idea in ss_pa.md is sound; it is one wire).

P8 — Not changed, confirm understood. With bias killed the IRF510s still conduct on gate RF peaks while DRIVER_BIAS is up; firmware must mute the envelope on FAULT_N as well. No hardware change.

## Output LPF board (KiCAD/Output LPF/)

Filter values are correct for 7-pole 0.1 dB Chebyshev and need no change.

L1 — Relay contact function (M15).

- ☐ Open the Omron G6K-T1 PDF bottom-view drawing and confirm which of pins 2/7 and 4/5 are NC vs NO. The wiring needs 4 and 5 = NO (RF busses), 2 and 7 = NC (swamps). If that holds, fix the Relay_G6K symbol description text; if it does not, swap the bus and swamp nets on every relay.

- ☐ Any G6K-2P DC12 (plain or -Y) is acceptable: 1.315 kΩ coil, 9.1 mA. Update the BOM note.

L2 — Coil-driver gate conditioning (S7).

- ☐ On each of Q1, Q5, Q8, Q10: 1 k series from the J2 pin to the gate, 100 k gate to GND. Keeps gates defined with the umbilical unplugged and knocks RF off the cable.

L3 — Parts cleanup.

- ☐ All CFx: specify 100 V C0G in the BOM field (shunt caps see ~0.8 A RF and up to 64 V peak into an open).

- ☐ Remove the "2 in parallel to reach" text from CF7, CF8, CF12, CF16 (values are single parts).

- ☐ L6–L20 Description fields: T68-6 → T50-6 to match the footprints, or change the footprints if you take the T68-6 option (turns drop ~15 %).

L4 — Firmware interlock (no hardware).

- ☐ Band change only with key up and envelope at zero; G6K contacts are not rated for hot switching at 10 W.

## Frontpanel board (KiCAD/frontpanel/)

The netlist has not changed since round 1; M11–M14 are all here.

F1 — Umbilical ground (M11).

- ☐ J1 pin 5: ENC_INT_CLEAN → GND, matching front_panel_interface.md (pair 1 = +5 V / GND). GND is then on a conductor, not just the shield.

F2 — Backlight FETs (M12). For each colour, move the resistor from source to drain:

- ☐ Red: LCD pin 16 → R1 200 Ω → Q1 drain; Q1 source → GND.

- ☐ Green: LCD pin 17 → R2 120 Ω → Q2 drain; Q2 source → GND.

- ☐ Blue: LCD pin 18 → R3 120 Ω → Q3 drain; Q3 source → GND.

- ☐ Expected currents ≈ 14 mA each (red V_f ≈ 2.0 V, G/B ≈ 3.3 V from 5 V). Raise to 100 Ω if the panel is dim; the module's rated max is 25–30 mA per colour.

F3 — Remove the ENC_INT path (M13).

- ☐ Delete J4 (the four shorted pins), R4, R5 and the ENC_INT_RAW / ENC_INT_CLEAN nets.

- ☐ U4 (3.3 V LDO) and C8 then power nothing: delete them, or keep U4 only if you want a 3.3 V tap for a future part. The MBL-600, PCF8575, MCP4725 and LCD are all on +5 V.

- ☐ PCF8575 IRQ stays no-connect; firmware polls per front_panel_interface.md.

F4 — Buttons (nice-to-have).

- ☐ J2: change to a 4-pin header with pin 4 = GND so the P13–P15 buttons have a return on the same connector.

F5 — Footprint (blocking for fab).

- ☐ Close the RJE1D-188-21401 fit check from the 2026-07-15 note: dry-fit a jack on the Rev A analog board, correct Amphenol_RJE1D-188_Horizontal_Shielded.kicad_mod once, and re-import on both analog and frontpanel.

F6 — Shield tabs. The netlist ties J1 SH to GND; the doc says leave it floating. Follow whatever you decide under Decisions and make the netlist and doc agree.

## Protoshield (KiCAD/protoshield/)

PS1 — Record the Metro pin assignments (M1). Place the Metro_ESP32S3 symbol and wire it so the schematic documents the hand-wiring. Proposed map using the names already in the symbol; change to taste, then mirror in pin_map.h:

| Metro pin | Signal | Via |
| --- | --- | --- |
| D2 (FREQ_A) | MBL600_A | R13 |
| D3 (FREQ_B) | MBL600_B | R14 |
| D4 (DIT) | PADDLE_A | new 1 k (PS4) |
| D5 (DAH) | PADDLE_B | new 1 k (PS4) |
| D6 | DAC_LDAC_N | J17 pin 5 |
| D7 (ADC1) | Source_Sense_A | R1 |
| D8 (ADC1) | Source_Sense_B | R2 |
| D9 (ADC1) | TR_SENSE | R10 pull-up |
| D10 | CS_DAC | J17 pin 4 |
| D11 | MOSI | J17 pin 3 |
| D13 | SCK | J17 pin 2 |
| D12 | FAULT_N_CLEAN | new 1 k (PS4) |
| A0 | LDAC_4728 | J1 pin 2 |
| A1, A2 | I_CATHODE_A/B (tube path, DNP) | R5, R6 |
| A3 | GRID_BLOCK_CRASH (DNP) | R9 |
| SDA, SCL | SDA_IN, SCL_IN | new J18 (PS2) |
| RESET | RESET_N | J1 pin 3 |
| 3V3 | +3.3V net (pull-ups) | wire |
| GND | GND | wire |

- ☐ Source_Sense on ADC1 pins (D7/D8), not ADC2: ADC2 is unusable while Wi-Fi is on.

PS2 — I²C terminal (N6).

- ☐ Add J18, 3-pin screw terminal: 1 GND, 2 SDA_IN, 3 SCL_IN. Analog J10 becomes 3-pin in the same order (A9).

PS3 — Encoder inputs (M14). After A7 the lines arrive at 3.0 V:

- ☐ Delete D3, D4.

- ☐ C8, C9: 100 nF → 100 pF (or delete). Edge time with R13/R14 1 k becomes < 1 µs.

PS4 — Contact and logic inputs (S9).

- ☐ PADDLE_A/B: R11, R12 10 k → 4.7 k; add 1 k series between J6 and the Metro pin; add 10 nF from the J6 side to GND.

- ☐ TR_SENSE: same treatment (1 k series, 10 nF at J5).

- ☐ FAULT_N_CLEAN: 1 k series between J8 pin 3 and the Metro pin.

- ☐ RESET_N: C1 100 nF is fine; add 1 k series if the button cable is long.

PS5 — Terminal order (S8).

- ☐ J17: GND, SCK, MOSI, CS_DAC, DAC_LDAC_N (currently GND, LDAC, SCK, MOSI, CS). See the harness table.

PS6 — Source_Sense (S10). R1/R2 1 k + C2/C3 10 nF are correct and sufficient once P2 is done on ss_pa; no zener needed. Update the handoff text that claims one.

PS7 — +3.3V net. It is the Metro's 3V3 pin. Say so on the sheet with a power-flag or the Metro symbol from PS1.

## Harness: analog ↔ protoshield terminals, made 1:1

Every terminal pair below should have the same pin count and order on both boards so a straight cable works. Rows marked change need a schematic edit; the rest already match.

| Function | Analog terminal (pin order) | Protoshield terminal (pin order) | Action |
| --- | --- | --- | --- |
| SPI to MCP4922 | J16: GND, SCK, MOSI, CS_DAC, DAC_LDAC_N | J17: GND, SCK, MOSI, CS_DAC, DAC_LDAC_N | change J17 order (PS5) |
| I²C from Metro | J10: GND, SDA_IN, SCL_IN | J18 (new): GND, SDA_IN, SCL_IN | change J10 to 3-pin; add J18 (PS2) |
| Encoder + MCP4728 LDAC | J12: GND, MBL600_B, MBL600_A, LDAC_4728 | J2 → 4-pin: GND, MBL600_B, MBL600_A, LDAC_4728 | change J2 to 4-pin; drop LDAC_4728 from J1 |
| Reset | J1 (shield): GND, RESET_N | — (button only) | change J1 to 2-pin after LDAC moves |
| PA sense + fault | J3: GND, Source_Sense_A, Source_Sense_B, FAULT_N_CLEAN | J8: GND, Source_Sense_A, Source_Sense_B, FAULT_N_CLEAN | change both: drop PA_Bias_x_Raw from J3 (TP3/TP4 cover probing), move FAULT_N_CLEAN from J14 |
| KeyUp | J14: GND, KeyUp | J19 (new): GND, KeyUp | change J14 to 2-pin; add J19 if the Metro drives KeyUp directly, else delete J14 and let PCF8575 P8 own it |
| Paddle | — | J6: PADDLE_A, PADDLE_B, GND | no change (paddle jack cable) |
| T/R sense | — | J5: GRID_BLOCK_CRASH, TR_SENSE | change split: J5 → GND, TR_SENSE; GRID_BLOCK_CRASH moves to J3 (tube path) |
| Tube path (DNP) | J22: GND, GRID_BLOCK_CRASH, I_CATHODE_B, I_CATHODE_A | J3 → 4-pin: GND, GRID_BLOCK_CRASH, I_CATHODE_B, I_CATHODE_A | change J3 to 4-pin, DNP both ends |
| Power out | J13: GND, +5V, +12V | — | decide Metro power (Decisions) |

- ☐ After editing, print this table into revD_interconnect.md as the authoritative harness map and delete the J31 "TBD" section there (J31 is fully assigned in both netlists).

RJ45 umbilicals already match end to end and need no pin changes except frontpanel/analog pin 5 → GND (A8, F1).

## Documentation corrections (same commit as the schematic edits)

revD_prelayout_review_handoff.md

- ☐ Topology: delete the "Chebyshev LPF (small, on-analog)" and "LM7171 push-pull driver (U4/U5)" items; describe R1/L1/C1 as the 28 MHz 2-pole filter and U1 as a single LM7171 driving ss_pa J3 single-ended.

- ☐ DAC assignment: MCP4922 VoutA = DRIVER_BIAS, VoutB = envelope (G2). Swap the text.

- ☐ Fault chain: replace "~18 µs" with the post-P4 figure (~220 µs); R9 = 4.7 k, not 10 k.

- ☐ Protoshield: R7 is 330 Ω; Source_Sense has an RC, not a zener; the 1 k series parts go on the paddles after PS4.

- ☐ Umbilical pin lists for J2 and J31: copy from the netlists (J2 pins 2–5 = RLY_15/17/20/40, pin 1 NC; J31 as wired).

- ☐ Frontpanel: R2/R3 are backlight resistors, not RS-422 terminators; RED/GREEN/BLUE come from the frontpanel PCF8575.

ss_pa.md

- ☐ Rewrite "Design intent" and "Signal interface": single-ended J3 → T201 → BS170 Q2/Q6 → T202 → IRF510; DRIVER_BIAS on J31 pin 6; connector is J31 not J7; relay is K5 not K6; T203 is ferrite (43 mix), not powdered iron.

- ☐ Replace "Response time <1 µs" with the P4 analysis.

- ☐ Source-sense bypass: 1 µF, plus the P2 RC at the comparator.

- ☐ Shield grounding paragraph: match the Decisions outcome.

front_panel_interface.md

- ☐ Strip the QT-encoder, J50/R102, STEMMA-chain and "BSS138 not needed" passages into a dated history section; keep the pin map, termination and polling decision.

- ☐ Paddle Path 1 now lives on the protoshield (J6), not analog J10.

- ☐ Shield-tab paragraph: match the Decisions outcome.

i2c_bus.md

- ☐ Drop the "depopulate breakout pull-ups" instruction; record the six-pull-up total (~1.7 kΩ, ~3 mA) as accepted.

- ☐ Note the SDA/SCL path physically goes Metro → protoshield J18 → analog J10 → U2.

revD_interconnect.md

- ☐ J2 pin 1 is NC (no RLY_10); pins 2–5 as in the netlist.

- ☐ J31 is assigned; delete the TX_ENABLE signal and the TBD block.

- ☐ J1: C4 is the DC block (C1 is shunt in the filter).

- ☐ LPF coils are G6K, not reeds; +12 V reaches the LPF on J2 pin 8.

- ☐ Add the harness table from this plan.

## Decisions only you can make

| Decision | Options | Recommendation |
| --- | --- | --- |
| Shield grounding policy | (a) both ends, as every netlist does; (b) single-point per the docs | (a). One chassis, HF, cables under 1 m: a shield grounded at both ends is a lower-impedance return and no loop problem. Make the docs say so. |
| −12 V rail | keep LT1054 (fix R17, A3) or go single-supply | Keep it. The simulated key-up fix (A2) needs a −3 V clamp; single-supply would need the unsimulated series-FET kill instead. |
| DGFET key-up method | clamp G2 to −3 V (A2, simulated), or series BS170 source kill (A2 alternative) | The −3 V clamp. G1-divider-only was simulated and leaks 5 mA; do not use it alone. |
| SW1 | header + shunts, or verified slide switch | Header + shunts. |
| Metro power on-air | USB (per front_panel_interface.md), or analog J21 polyfuse → barrel jack at +12 V, or analog J13 +5 V → Metro 5 V pin | J21 → barrel jack. One supply, no wall wart. Add the Metro's ~200 mA to the +12 V budget and confirm the J21 polyfuse rating. |
| Protoshield build | hand-wired Adafruit #2077, or fabricated PCB | If PCB: the Metro header footprint must be in the netlist, not just the symbol. |
| KeyUp ownership | PCF8575 P8 only, or Metro GPIO via J14/J19 | PCF8575 only; delete J14 and J19. Key-up timing is handled by the envelope DAC, not by this mute. |
| LPF relay logic polarity | active-high (all four banks energise at PCF8575 power-on), or active-low | Active-high is acceptable with L2's pull-downs and K5 holding the PA off at boot; firmware writes the PCF8575 before enabling K5. |
| RJE1D-188 footprint | fit-checked on Rev A, or not yet | Must be closed before either RJ45 board goes to fab (F5). |

## Pre-layout verification checklist

Run this after the edits above, in order. If any item fails, fix it before placing a footprint.

- ☐ Regenerate all five netlists and diff against the 2026-10-23 set; every change should trace to an item in this plan.

- ☐ ERC clean on all five projects with no suppressed warnings; every symbol has a footprint and a value (grep the netlists for empty Footprint fields).

- ☐ Analog: confirm no net connects a 5 V output to a Metro pin without a divider or shifter (FAULT_N_CLEAN, MBL600_A/B, SDA/SCL via U2 only).

- ☐ Analog: QUCS-S DC operating point of Q2 with the A2 divider: I_D 10–14 mA keyed, drain 5–7 V, source 2.5–3 V; key-up (G2 clamped to −3 V) I_D < 1 µA, V_G2S ≤ −2.5 V.

- ☐ Analog: U1 pin 3 DC = 0 V; output DC within 50 mV of it; −12 V rail ≥ 9.5 V magnitude under keyed load.

- ☐ ss_pa: LM2936Z pin map checked against the TI drawing, +5 V output cap ≥ 10 µF; LM393 −in nodes have the 1 k/10 nF/220 k network; R12/R19 = 2.2 k.

- ☐ ss_pa: hand-trace the latch: FAULT_N = 0 → V_REF = 0 V while −in ≥ 23 mV → stays tripped; J5 short → releases.

- ☐ LPF: relay NO/NC confirmed from the datasheet drawing; gate 1 k/100 k present on all four drivers.

- ☐ Frontpanel: J1 pin 5 = GND; R1–R3 in the drain legs; no ENC_INT nets remain.

- ☐ Protoshield: Metro symbol wired; every screw terminal in the harness table has a matching partner with identical pin order; no single-node nets except deliberate DNP paths.

- ☐ Harness table pasted into revD_interconnect.md; umbilical pin lists in the handoff regenerated from the netlists.

- ☐ Power budget written down per rail with the Metro included: raw bus (PA + regulators), +12 V, +5 V, −12 V (if kept); U6 heatsink sized to it.

- ☐ RJE1D-188 fit check closed and footprint re-imported on analog and frontpanel.

- ☐ Firmware notes captured for later: K5 delay ≥ 10 s (or 1 s with 100 Ω), PCF8575 written before K5 closes, envelope muted on FAULT_N, band change key-up only, MCP4728 reprogram with Si5351 shunts pulled.